# JJM Experiment Summary

- Dataset –

  - We converted and cleaned the dataset to get 677 datapoints from 22 experiments.

  - Input Format: Individual readings(in m^3/min) of each flow meters.

  - Output Format: If leak is present (L/M/H/OFF) in L1, L3, L4 respectively.

  - IMP: fm2891 showed almost no change in flow rate across experiments. There are chances that the meter is malfunctioned.

  - Note: Due to the repetitive nature of the data, it is tough to create a homogenous train/test person.

  - Note: The data generated for every flow meter has very few (< 10) unique values - there is very little to no variation in the water flow rate (m^3/min) in the experiments.

- Models Tried –

  - With the dataset available it was difficult to train any model as the models tend to overfit the data.

  - We will use a supervised learning approach. We trained a binary classifier model per leakage (i.e. the pipe) as sufficient amount of data from L/M/H classes wasn’t available.

  - We tried using Decision tree, Random forest and Logistic Regression by modelling the prediction as a binary classification problem.

  - We did not get significant results due to the poor quality data.

- Results – L1 was detected better most of the times as most probably the flow meters at both its ends were working properly.

| Model | L1 % Accuracy | L2 % Accuracy | L3 % Accuracy |
| --- | --- | --- | --- |
| Decision Tree | 63.97 | 57.36 | 54.41 |
| Random Forest | 65.44 | 57.36 | 54.41 |
| Logistic Regression | 57.35 | 52.21 | 58.08 |

- Future direction – Additional sensors (for example, accelerometers) with a better precision and sampling frequency need to be added. The data collection has to be more fine-grained. The experiments will need to be redone and it will be better is the transient information also needs to be captured for time series analytics.